1, Set up a smart home intrusion detection system using PIR and door/window sensors. LED and buzzer alert on unauthorized access. Show the IoT setup in Tinkercad.



int pirPin = 2;

int doorPin = 3;

int ledPin = 10;

int buzzerPin = 11;

void setup() {

pinMode(pirPin, INPUT);

pinMode(doorPin, INPUT_PULLUP);

pinMode(ledPin, OUTPUT);

pinMode(buzzerPin, OUTPUT);

}

void loop() {

bool motion = digitalRead(pirPin) == HIGH;

bool doorOpen = digitalRead(doorPin) == LOW;

if (motion || doorOpen) {

digitalWrite(ledPin, HIGH);

tone(buzzerPin, 1000);

} else {

digitalWrite(ledPin, LOW);

noTone(buzzerPin);

}

}




2. Create a smart laboratory chemical leak alert using gas sensors. LED and buzzer alert on hazardous gas detection. Present the IoT design in Tinkercad.

const int gasPin = A0;

const int ledPin = 10;

const int buzzerPin = 11;

int threshold = 400; // adjust for sensitivity

void setup() {

pinMode(ledPin, OUTPUT);

pinMode(buzzerPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int value = analogRead(gasPin);

Serial.println(value);

if (value > threshold) {

digitalWrite(ledPin, HIGH);

tone(buzzerPin, 1500);

} else {

digitalWrite(ledPin, LOW);

noTone(buzzerPin);

}

delay(200);

}


3. Implement a smart server power alert system using AC current sensors. LED and buzzer alert on high usage or abnormal activity. Show the IoT architecture in Tinkercad.





const int sensorPin = A0;

const int ledGreen = 2;

const int ledYellow = 3;

const int ledRed = 4;

const int buzzer = 5;

int currentValue = 0;

float amps = 0;

void setup() {

Serial.begin(9600);

pinMode(ledGreen, OUTPUT);

pinMode(ledYellow, OUTPUT);

pinMode(ledRed, OUTPUT);

pinMode(buzzer, OUTPUT);

}

void loop() {

currentValue = analogRead(sensorPin);

amps = currentValue * (30.0 / 1023.0);

if (amps < 10) {

digitalWrite(ledGreen, HIGH);

digitalWrite(ledYellow, LOW);

digitalWrite(ledRed, LOW);

digitalWrite(buzzer, LOW);

}

else if (amps >= 10 && amps < 18) {

digitalWrite(ledGreen, LOW);

digitalWrite(ledYellow, HIGH);

digitalWrite(ledRed, LOW);

digitalWrite(buzzer, LOW);

}

else {

digitalWrite(ledGreen, LOW);

digitalWrite(ledYellow, LOW);

digitalWrite(ledRed, HIGH);

digitalWrite(buzzer, HIGH);

}

Serial.print("Current: ");

Serial.print(amps);

Serial.println(" A");

delay(300);

}



4. Set up a smart warehouse motion detection system using PIR sensors. LED and buzzer alert on unauthorized movement. Show the IoT architecture in Tinkercad.

 

const int pirPin = 7;

const int led = 2;

const int buzzer = 3;

int motionState = 0;

void setup() {

pinMode(pirPin, INPUT);

pinMode(led, OUTPUT);

pinMode(buzzer, OUTPUT);

Serial.begin(9600);

}

void loop() {

motionState = digitalRead(pirPin);

if (motionState == HIGH) {

digitalWrite(led, HIGH);

digitalWrite(buzzer, HIGH);

Serial.println("Motion Detected");

} else {

digitalWrite(led, LOW);

digitalWrite(buzzer, LOW);

}

delay(200);

}


5. Develop a smart server room door monitor using magnetic sensors. LED and buzzer alert on unauthorized opening. Describe the IoT structure in Tinkercad.



const int doorPin = 7;

const int led = 2;

const int buzzer = 3;

int doorState = 0;

void setup() {

pinMode(doorPin, INPUT_PULLUP);

pinMode(led, OUTPUT);

pinMode(buzzer, OUTPUT);

Serial.begin(9600);

}

void loop() {

doorState = digitalRead(doorPin);

if (doorState == LOW) {

digitalWrite(led, HIGH);

digitalWrite(buzzer, HIGH);

Serial.println("Unauthorized Door Opening");

} else {

digitalWrite(led, LOW);

digitalWrite(buzzer, LOW);

}

delay(200);

}


6. Implement a smart home energy monitor using AC current sensors. LED and buzzer alert on high consumption. Describe the IoT structure in Tinkercad.





const int sensorPin = A0;

const int ledGreen = 2;

const int ledYellow = 3;

const int ledRed = 4;

const int buzzer = 5;

int currentValue = 0;

float amps = 0;

void setup() {

Serial.begin(9600);

pinMode(ledGreen, OUTPUT);

pinMode(ledYellow, OUTPUT);

pinMode(ledRed, OUTPUT);

pinMode(buzzer, OUTPUT);

}

void loop() {

currentValue = analogRead(sensorPin);

amps = currentValue * (30.0 / 1023.0);

if (amps < 10) {

digitalWrite(ledGreen, HIGH);

digitalWrite(ledYellow, LOW);

digitalWrite(ledRed, LOW);

digitalWrite(buzzer, LOW);

}

else if (amps >= 10 && amps < 18) {

digitalWrite(ledGreen, LOW);

digitalWrite(ledYellow, HIGH);

digitalWrite(ledRed, LOW);

digitalWrite(buzzer, LOW);

}

else {

digitalWrite(ledGreen, LOW);

digitalWrite(ledYellow, LOW);

digitalWrite(ledRed, HIGH);

digitalWrite(buzzer, HIGH);

}

Serial.print("Current: ");

Serial.print(amps);

Serial.println(" A");

delay(300);

}


7. Implement a smart parking lot security system using PIR and ultrasonic sensors. LED and buzzer alert on unauthorized access. Present the IoT design in Tinkercad.



const int pirPin = 7;

const int trigPin = 8;

const int echoPin = 9;

const int led = 2;

const int buzzer = 3;

long duration;

int distance;

int pirState = 0;

void setup() {

pinMode(pirPin, INPUT);

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(led, OUTPUT);

pinMode(buzzer, OUTPUT);

Serial.begin(9600);

}

void loop() {

pirState = digitalRead(pirPin);

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH);

distance = duration * 0.034 / 2;

if (pirState == HIGH || distance < 50) {

digitalWrite(led, HIGH);

digitalWrite(buzzer, HIGH);

Serial.println("Unauthorized Access Detected");

} else {

digitalWrite(led, LOW);

digitalWrite(buzzer, LOW);

}

delay(200);

}



8. Create a smart ATM monitoring system using PIR and magnetic sensors. LED and buzzer alert on tampering or unauthorized access. Explain the IoT layout in Tinkercad.



const int pirPin = 7;

const int magPin = 8;

const int led = 2;

const int buzzer = 3;

int pirState = 0;

int magState = 0;

void setup() {

pinMode(pirPin, INPUT);

pinMode(magPin, INPUT_PULLUP);

pinMode(led, OUTPUT);

pinMode(buzzer, OUTPUT);

Serial.begin(9600);

}

void loop() {

pirState = digitalRead(pirPin);

magState = digitalRead(magPin);

if (pirState == HIGH || magState == LOW) {

digitalWrite(led, HIGH);

digitalWrite(buzzer, HIGH);

Serial.println("Unauthorized ATM Access Detected");

} else {

digitalWrite(led, LOW);

digitalWrite(buzzer, LOW);

}

delay(200);

}

9. Create a smart corridor fire alert system using smoke and flame sensors. LED and buzzer alert on fire detection. Describe the IoT structure in Tinkercad.

int smokePin = 2;

int flamePin = 3;

int ledPin = 8;

int buzzerPin = 9;

void setup() {

pinMode(smokePin, INPUT);

pinMode(flamePin, INPUT);

pinMode(ledPin, OUTPUT);

pinMode(buzzerPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int smokeState = digitalRead(smokePin);

int flameState = digitalRead(flamePin);

if(smokeState == HIGH || flameState == HIGH){

digitalWrite(ledPin, HIGH);

digitalWrite(buzzerPin, HIGH);

Serial.println("FIRE ALERT!");

} else {

digitalWrite(ledPin, LOW);

digitalWrite(buzzerPin, LOW);

}

delay(200);

}